# 基于FRDM-KE02Z40M的ucos-ii移植

作者：Jianhui Tong

邮箱：Jianhui.tong@nxp.com

# 第一部分 准备工作

- 下载FRDM-KEXX Driver Library Package，即KEXX_DRIVERS_V1.2.1_DEVD.zip文件。

下载链接为：

http://www.nxp.com/webapp/sps/download/license.jsp?colCode=KEXX_DRIVERS_V1.2.1_DEVD&location=null&fromsite=zh-Hans&fsrch=1&sr=1&pageNum=1&Parent_nodeId=&Parent_pageType

- 下载KL02 SDK2.0，下载时需要勾选ucos-ii。

访问Kinetis Expet网站：http://kex.nxp.com/en/welcome，点击“Build an SDK”。

选择”Configuration”标签页，并点击”New Configuration”新建配置。

选择FRDM-KL02Z开发板，并点击”Select Configuration”。

勾选ucos-ii，并点击”Build SDK Package”。

切换到”Software Vault”标签页，并下载刚刚Build完成的SDK。

- 将下载好的KE驱动库和SDK库解压

- 运行..\KEXX_DRIVERS_V1.2.1_DEVD\kexx_drv_lib\build\iar\ke02目录下的make_new_project_ke02.exe文件。

- 输入新建工程名字，如ucosii_ke02，回车确认。

- 新建完成后，再次按下回车键退出。

- 打开..\KEXX_DRIVERS_V1.2.1_DEVD\kexx_drv_lib\build\iar\ke02\ucosii_ke02目录下的ucosii_ke02.eww文件，并编译工程。

- 此时，即得到了使用KE driver库的Demo工程，后面移植工作将基于此工程进行修改。

# 第二部分 移植ucos-ii

- ..\KEXX_DRIVERS_V1.2.1_DEVD\kexx_drv_lib\src\projects\KE02\ucosii_ke02

目录下新建两个文件夹，并分别命名为“ports”，“uCOS-II”。

- 将..\SDK_2.0_FRDM-KL02Z\rtos\ucosii_2.92.11\uCOS-II\Ports\ARM-Cortex-M0\Generic\IAR目录下的4个文件 os_cpu.h, os_cpu_a.asm, os_cpu_c.c, os_dbg.c复制到

..\KEXX_DRIVERS_V1.2.1_DEVD\kexx_drv_lib\src\projects\KE02\ucosii_ke02\ports

- 将..\SDK_2.0_FRDM-KL02Z\rtos\ucosii_2.92.11\uCOS-II\Source目录下的11个文件os_core.c, os_flag.c, os_mbox.c, os_mem.c, os_mutex.c, os_q.c, os_sem.c, os_task.c, os_time.c, os_tmr.c, ucos_ii.h复制到

..\KEXX_DRIVERS_V1.2.1_DEVD\kexx_drv_lib\src\projects\KE02\ucosii_ke02\uCOS-II

- 将..\SDK_2.0_FRDM-KL02Z\rtos\ucosii_2.92.11\template_application目录下的4个文件app_cfg.h, app_hooks.c, cpu_cfg.h, os_cfg.h 复制到

..\KEXX_DRIVERS_V1.2.1_DEVD\kexx_drv_lib\src\projects\KE02\ucosii_ke02

在IAR工程目录中新建两个文件夹“ports”和“ucos ii”。

- 将上述步骤中复制的文件添加到IAR工程中，如下图所示。

- 修改编译器搜索路径。在IAR中打开Project > Options > C/C++ Compiler >Preprocessor，在“Additional include directories”一栏中添加上文复制文件所在的路径。

- 修改app_cfg.h文件，添加下图红框中圈出的代码。

- 修改ucosii_ke02.c文件，用下面的代码替换。

/****************************************************************************** * * Freescale Semiconductor Inc. * (c) Copyright 2013 Freescale Semiconductor, Inc. * ALL RIGHTS RESERVED. * *************************************************************************** * * THIS SOFTWARE IS PROVIDED BY FREESCALE "AS IS" AND ANY EXPRESSED OR * IMPLIED WARRANTIES, INCLUDING, BUT NOT LIMITED TO, THE IMPLIED WARRANTIES * OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE ARE DISCLAIMED. * IN NO EVENT SHALL FREESCALE OR ITS CONTRIBUTORS BE LIABLE FOR ANY DIRECT, * INDIRECT, INCIDENTAL, SPECIAL, EXEMPLARY, OR CONSEQUENTIAL DAMAGES * (INCLUDING, BUT NOT LIMITED TO, PROCUREMENT OF SUBSTITUTE GOODS OR * SERVICES; LOSS OF USE, DATA, OR PROFITS; OR BUSINESS INTERRUPTION) * HOWEVER CAUSED AND ON ANY THEORY OF LIABILITY, WHETHER IN CONTRACT, * STRICT LIABILITY, OR TORT (INCLUDING NEGLIGENCE OR OTHERWISE) ARISING * IN ANY WAY OUT OF THE USE OF THIS SOFTWARE, EVEN IF ADVISED OF * THE POSSIBILITY OF SUCH DAMAGE. * ***************************************************************************//*! * * @file uCOS-II_KE02.c * * @author Freescale * * @version 0.0.1 * * @date Jun. 25, 2013 * * @brief providing framework of demo cases for MCU. * *******************************************************************************/ #include "common.h" #include "ics.h" #include "uart.h" #include "sysinit.h" #include "ucos_ii.h" #include "system_MKE02Z2.h" /****************************************************************************** * Global variables ******************************************************************************/ /****************************************************************************** * Constants and macros ******************************************************************************/ /****************************************************************************** * Local types ******************************************************************************/ /****************************************************************************** * Local function prototypes ******************************************************************************/ /****************************************************************************** * Local variables ******************************************************************************/ static OS_STK AppTaskStartStk[APP_CFG_TASK_START_STK_SIZE]; static OS_STK AppTaskLedRStk[APP_CFG_TASK_LED_R_STK_SIZE]; static OS_STK AppTaskLedGStk[APP_CFG_TASK_LED_G_STK_SIZE]; static OS_STK AppTaskLedBStk[APP_CFG_TASK_LED_B_STK_SIZE]; static OS_EVENT* RSem; static OS_EVENT* GSem; static OS_EVENT* BSem; static OS_EVENT* OffSem; /****************************************************************************** * Local functions ******************************************************************************/ int main (void); static void AppTaskStart (void *p_arg); static void AppTaskLedR(void *p_arg); static void AppTaskLedG(void *p_arg); static void AppTaskLedB(void *p_arg); /****************************************************************************** * Global functions ******************************************************************************/ /********************************************************************/ int main (void) { OSInit(); OSTaskCreate(AppTaskStart, (void*)0, &AppTaskStartStk[APP_CFG_TASK_START_STK_SIZE-1], (INT8U)APP_CFG_TASK_START_PRIO ); OSStart(); return (1); } static void AppTaskStart (void *p_arg) { INT8U err; (void)p_arg; sysinit(); OS_CPU_SysTickInit(20000000/OS_TICKS_PER_SEC); PORT->HDRVE ¦=0x02; LED0_Init(); LED1_Init(); LED2_Init(); LED0_Off(); LED1_Off(); LED2_Off(); RSem = OSSemCreate(1); GSem = OSSemCreate(1); BSem = OSSemCreate(1); OffSem = OSSemCreate(1); OSTaskCreate(AppTaskLedR, (void*)0, &AppTaskLedRStk[APP_CFG_TASK_LED_R_STK_SIZE-1], (INT8U)APP_CFG_TASK_LED_R_PRIO ); OSTaskCreate(AppTaskLedG, (void*)0, &AppTaskLedGStk[APP_CFG_TASK_LED_G_STK_SIZE-1], (INT8U)APP_CFG_TASK_LED_G_PRIO ); OSTaskCreate(AppTaskLedB, (void*)0, &AppTaskLedBStk[APP_CFG_TASK_LED_B_STK_SIZE-1], (INT8U)APP_CFG_TASK_LED_B_PRIO ); while (1) { OSTimeDlyHMSM(0,0,3,0); OSSemPost(RSem); OSSemPend(OffSem, 0, &err); } } static void AppTaskLedR(void *p_arg) { INT8U err; while(1) { OSSemPend(RSem,0,&err); LED0_On();LED2_Off(); OSTimeDlyHMSM(0,0,1,0); OSSemPost(GSem); } } static void AppTaskLedG(void *p_arg) { INT8U err; while(1) { OSSemPend(GSem,0,&err); LED1_On();LED0_Off(); OSTimeDlyHMSM(0,0,1,0); OSSemPost(BSem); } } static void AppTaskLedB(void *p_arg) { INT8U err; while(1) { OSSemPend(BSem,0,&err); LED2_On();LED1_Off(); OSTimeDlyHMSM(0,0,1,0); OSSemPost(OffSem); } }

- 修改os_cfg.h文件，用下面的代码替换。（由于KE02的RAM较小，因此关闭了部分未使用的功能）

/* ********************************************************************************************************* * uC/OS-II * The Real-Time Kernel * uC/OS-II Configuration File for V2.9x * * (c) Copyright 2005-2013, Micrium, Weston, FL * All Rights Reserved * * * File : OS_CFG.H * By : Jean J. Labrosse * Version : V2.92.11 * * LICENSING TERMS: * --------------- * uC/OS-II is provided in source form for FREE evaluation, for educational use or for peaceful research. * If you plan on using uC/OS-II in a commercial product you need to contact Micrium to properly license * its use in your product. We provide ALL the source code for your convenience and to help you experience * uC/OS-II. The fact that the source is provided does NOT mean that you can use it without paying a * licensing fee. ********************************************************************************************************* */ #ifndef OS_CFG_H #define OS_CFG_H /* ---------------------- MISCELLANEOUS ----------------------- */ #define OS_APP_HOOKS_EN 0u /* Application-defined hooks are called from the uC/OS-II hooks */ #define OS_ARG_CHK_EN 0u /* Enable (1) or Disable (0) argument checking */ #define OS_CPU_HOOKS_EN 1u /* uC/OS-II hooks are found in the processor port files */ #define OS_DEBUG_EN 0u /* Enable(1) debug variables */ #define OS_EVENT_MULTI_EN 0u /* Include code for OSEventPendMulti() */ #define OS_EVENT_NAME_EN 0u /* Enable names for Sem, Mutex, Mbox and Q */ #define OS_LOWEST_PRIO 63u /* Defines the lowest priority that can be assigned ... */ /* ... MUST NEVER be higher than 254! */ #define OS_MAX_EVENTS 10u /* Max. number of event control blocks in your application */ #define OS_MAX_FLAGS 5u /* Max. number of Event Flag Groups in your application */ #define OS_MAX_MEM_PART 5u /* Max. number of memory partitions */ #define OS_MAX_QS 4u /* Max. number of queue control blocks in your application */ #define OS_MAX_TASKS 10u /* Max. number of tasks in your application, MUST be >= 2 */ #define OS_SCHED_LOCK_EN 1u /* Include code for OSSchedLock() and OSSchedUnlock() */ #define OS_TICK_STEP_EN 1u /* Enable tick stepping feature for uC/OS-View */ #define OS_TICKS_PER_SEC 1000u /* Set the number of ticks in one second */ #define OS_TLS_TBL_SIZE 0u /* Size of Thread-Local Storage Table */ /* --------------------- TASK STACK SIZE ---------------------- */ #define OS_TASK_TMR_STK_SIZE 64u /* Timer task stack size (# of OS_STK wide entries) */ #define OS_TASK_STAT_STK_SIZE 64u /* Statistics task stack size (# of OS_STK wide entries) */ #define OS_TASK_IDLE_STK_SIZE 64u /* Idle task stack size (# of OS_STK wide entries) */ /* --------------------- TASK MANAGEMENT ---------------------- */ #define OS_TASK_CHANGE_PRIO_EN 1u /* Include code for OSTaskChangePrio() */ #define OS_TASK_CREATE_EN 1u /* Include code for OSTaskCreate() */ #define OS_TASK_CREATE_EXT_EN 1u /* Include code for OSTaskCreateExt() */ #define OS_TASK_DEL_EN 0u /* Include code for OSTaskDel() */ #define OS_TASK_NAME_EN 0u /* Enable task names */ #define OS_TASK_PROFILE_EN 0u /* Include variables in OS_TCB for profiling */ #define OS_TASK_QUERY_EN 0u /* Include code for OSTaskQuery() */ #define OS_TASK_REG_TBL_SIZE 1u /* Size of task variables array (#of INT32U entries) */ #define OS_TASK_STAT_EN 0u /* Enable (1) or Disable(0) the statistics task */ #define OS_TASK_STAT_STK_CHK_EN 0u /* Check task stacks from statistic task */ #define OS_TASK_SUSPEND_EN 0u /* Include code for OSTaskSuspend() and OSTaskResume() */ #define OS_TASK_SW_HOOK_EN 1u /* Include code for OSTaskSwHook() */ /* ----------------------- EVENT FLAGS ------------------------ */ #define OS_FLAG_EN 0u /* Enable (1) or Disable (0) code generation for EVENT FLAGS */ #define OS_FLAG_ACCEPT_EN 1u /* Include code for OSFlagAccept() */ #define OS_FLAG_DEL_EN 1u /* Include code for OSFlagDel() */ #define OS_FLAG_NAME_EN 1u /* Enable names for event flag group */ #define OS_FLAG_QUERY_EN 1u /* Include code for OSFlagQuery() */ #define OS_FLAG_WAIT_CLR_EN 1u /* Include code for Wait on Clear EVENT FLAGS */ #define OS_FLAGS_NBITS 16u /* Size in #bits of OS_FLAGS data type (8, 16 or 32) */ /* -------------------- MESSAGE MAILBOXES --------------------- */ #define OS_MBOX_EN 0u /* Enable (1) or Disable (0) code generation for MAILBOXES */ #define OS_MBOX_ACCEPT_EN 1u /* Include code for OSMboxAccept() */ #define OS_MBOX_DEL_EN 1u /* Include code for OSMboxDel() */ #define OS_MBOX_PEND_ABORT_EN 1u /* Include code for OSMboxPendAbort() */ #define OS_MBOX_POST_EN 1u /* Include code for OSMboxPost() */ #define OS_MBOX_POST_OPT_EN 1u /* Include code for OSMboxPostOpt() */ #define OS_MBOX_QUERY_EN 1u /* Include code for OSMboxQuery() */ /* --------------------- MEMORY MANAGEMENT -------------------- */ #define OS_MEM_EN 0u /* Enable (1) or Disable (0) code generation for MEMORY MANAGER */ #define OS_MEM_NAME_EN 1u /* Enable memory partition names */ #define OS_MEM_QUERY_EN 1u /* Include code for OSMemQuery() */ /* ---------------- MUTUAL EXCLUSION SEMAPHORES --------------- */ #define OS_MUTEX_EN 0u /* Enable (1) or Disable (0) code generation for MUTEX */ #define OS_MUTEX_ACCEPT_EN 1u /* Include code for OSMutexAccept() */ #define OS_MUTEX_DEL_EN 1u /* Include code for OSMutexDel() */ #define OS_MUTEX_QUERY_EN 1u /* Include code for OSMutexQuery() */ /* ---------------------- MESSAGE QUEUES ---------------------- */ #define OS_Q_EN 0u /* Enable (1) or Disable (0) code generation for QUEUES */ #define OS_Q_ACCEPT_EN 1u /* Include code for OSQAccept() */ #define OS_Q_DEL_EN 1u /* Include code for OSQDel() */ #define OS_Q_FLUSH_EN 1u /* Include code for OSQFlush() */ #define OS_Q_PEND_ABORT_EN 1u /* Include code for OSQPendAbort() */ #define OS_Q_POST_EN 1u /* Include code for OSQPost() */ #define OS_Q_POST_FRONT_EN 1u /* Include code for OSQPostFront() */ #define OS_Q_POST_OPT_EN 1u /* Include code for OSQPostOpt() */ #define OS_Q_QUERY_EN 1u /* Include code for OSQQuery() */ /* ------------------------ SEMAPHORES ------------------------ */ #define OS_SEM_EN 1u /* Enable (1) or Disable (0) code generation for SEMAPHORES */ #define OS_SEM_ACCEPT_EN 1u /* Include code for OSSemAccept() */ #define OS_SEM_DEL_EN 1u /* Include code for OSSemDel() */ #define OS_SEM_PEND_ABORT_EN 1u /* Include code for OSSemPendAbort() */ #define OS_SEM_QUERY_EN 1u /* Include code for OSSemQuery() */ #define OS_SEM_SET_EN 1u /* Include code for OSSemSet() */ /* --------------------- TIME MANAGEMENT ---------------------- */ #define OS_TIME_DLY_HMSM_EN 1u /* Include code for OSTimeDlyHMSM() */ #define OS_TIME_DLY_RESUME_EN 1u /* Include code for OSTimeDlyResume() */ #define OS_TIME_GET_SET_EN 1u /* Include code for OSTimeGet() and OSTimeSet() */ #define OS_TIME_TICK_HOOK_EN 1u /* Include code for OSTimeTickHook() */ /* --------------------- TIMER MANAGEMENT --------------------- */ #define OS_TMR_EN 0u /* Enable (1) or Disable (0) code generation for TIMERS */ #define OS_TMR_CFG_MAX 16u /* Maximum number of timers */ #define OS_TMR_CFG_NAME_EN 1u /* Determine timer names */ #define OS_TMR_CFG_WHEEL_SIZE 7u /* Size of timer wheel (#Spokes) */ #define OS_TMR_CFG_TICKS_PER_SEC 10u /* Rate at which timer management task runs (Hz) */ #endif

- 修改isr.h文件。

- 完成后保存，并重新编译工程。将下载程序到FRDM-KE02Z40M中，复位运行程序就可以看到LED灯的颜色随时间变化啦！